DIGITAL ELECTRONICS

LAB MANUAL

▸  Exp 1A — Basic Gates using NAND Gate (IC 7400) — Separate Circuit for Each Gate

▸  Exp 1B — Basic Gates using NOR Gate  (IC 7402) — Separate Circuit for Each Gate

▸  Exp 2  — K-Map: 3-Variable & 4-Variable (Full Calculation + 9 Solved Examples)

▸  Exp 3  — Parity Generator and Checker (IC 7486)

▸  Exp 4  — BCD to Excess-3 (K-Map + Full IC Circuit + IC Count)

▸  Exp 5A — Full Adder (K-Map Derivation + IC 7486/7408/7432 Circuit)

▸  Exp 5B — Full Subtractor (K-Map Derivation + Full IC Circuit)

Md Fazley Rabbi

IC Name & IC Number:

| IC Name | IC Number |
| --- | --- |
| AND Gate | - 7408: Quad 2-input AND (TTL) |
| OR Gate | - 7432: Quad 2-input OR (TTL) |
| NOT Gate (Inverter) | - 7404: Hex Inverter (TTL) |
| NAND Gate | - 7400: Quad 2-input NAND (TTL) |
| NOR Gate | - 7402: Quad 2-input NOR (TTL) |
| XOR Gate | - 7486: Quad 2-input XOR (TTL) |
| XNOR Gate | - 74266: Quad 2-input XNOR (TTL) |

# EXPERIMENT 1A: Basic Gates Using NAND Gate (IC 7400)

## Theory:

NAND gate: Y = (A·B)' = A' + B'  [De Morgan's Theorem]

NAND is called a Universal Gate because every other gate can be built from it alone. This is proven below by constructing NOT, AND, OR, and XOR entirely from NAND gates.

## IC 7400 — Quad 2-Input NAND — Full Pin Diagram

IC 7400 is a 14-pin DIP TTL IC containing FOUR independent 2-input NAND gates.

| IC 7400 — 7400 |  |  |
| --- | --- | --- |
| Pin 1 — A1 (Gate 1 Input A) | │ | Pin 14 — VCC (+5V) |
| Pin 2 — B1 (Gate 1 Input B) | │ | Pin 13 — A4 (Gate 4 Input A) |
| Pin 3 — Y1 (Gate 1 Output) | │ | Pin 12 — B4 (Gate 4 Input B) |
| Pin 4 — A2 (Gate 2 Input A) | │ | Pin 11 — Y4 (Gate 4 Output) |
| Pin 5 — B2 (Gate 2 Input B) | │ | Pin 10 — A3 (Gate 3 Input A) |
| Pin 6 — Y2 (Gate 2 Output) | │ | Pin 9 — B3 (Gate 3 Input B) |
| Pin 7 — GND (0V) | │ | Pin 8 — Y3 (Gate 3 Output) |
| Quad 2-input NAND |  |  |

RULE: Always connect Pin 14 to +5V and Pin 7 to GND before making other connections.

## Circuit 1A-1: NOT Gate from NAND (IC 7400)

Formula: Y = A'   |   NAND gates needed: 1   |   IC 7400 needed: 1

### Derivation:

Tie both inputs of one NAND gate together:

Y = NAND(A, A) = (A·A)' = A'     [since A·A = A by idempotent law]

### Truth Table:

| A (Input) | Y = A' (Output) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

## Circuit 1A-2: AND Gate from NAND (IC 7400)

Formula: Y = A·B   |   NAND gates needed: 2   |   IC 7400 needed: 1

### Derivation:

Step 1: W = NAND(A,B) = (A·B)'

Step 2: Y = NAND(W,W) = (W·W)' = W' = ((A·B)')' = A·B

### Truth Table:

| A | B | W = NAND(A,B) | Y = W' = A·B |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

** If Both value are true when it is true, otherwise false. **

## Circuit 1A-3: OR Gate from NAND (IC 7400)

Formula: Y = A+B   |   NAND gates needed: 3   |   IC 7400 needed: 1

### Derivation (De Morgan's Theorem):

A + B  =  (A'' + B'')  =  (A'·B')'   [De Morgan's]

Step 1: W1 = NAND(A,A) = A'          [Gate 1 as NOT]

Step 2: W2 = NAND(B,B) = B'          [Gate 2 as NOT]

Step 3: Y  = NAND(W1,W2) = (A'·B')' = A+B   [Gate 3]

### Truth Table:

| A | B | W1=A' | W2=B' | Y = (A'·B')' = A+B |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |

## Circuit 1A-4: XOR Gate from NAND (IC 7400)

Formula: Y = A xor B   |   NAND gates needed: 4   |   IC 7400 needed: 1

### Derivation (4-NAND minimum XOR):   A⊕B=A′B+AB′

X=(AB)′

Y=(A⋅X)′

Z=(B⋅X)′

F=(Y⋅Z)′

X = NAND(A,B)

Y = NAND(A,X)

Z = NAND(B,X)

P = NAND(Y,Z)      ← XOR

### Truth Table with Intermediate Signals:

| A | B | W1=(AB)' | W2=(A·W1)' | W3=(B·W1)' | Y=(W2·W3)' |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 |

## Circuit 1A-5: XNOR Gate from NAND (IC 7400)

Formula: Y = (A xor B)'   |   NAND gates needed: 5   |   IC 7400 needed: 2

XNOR = NOT(XOR). Build XOR using 4 NANDs (1 full IC 7400), then add Gate 5 as NOT on XOR output.

Gate 5: Y = NAND(XOR_out, XOR_out) = NOT(A xor B) = A xnor B

Since all 4 gates of IC 7400 #1 are used for XOR, Gate 5 must use a second IC 7400.

X = NAND(A,B)

Y = NAND(A,X)

Z = NAND(B,X)

P = NAND(Y,Z)      ← XOR

F = NAND(P,P)      ← XNOR

** XOR gate NOT korlei hoye jabe**

### Circuits ss:

### Gate: NOT, AND, OR

# EXPERIMENT 1B: Basic Gates Using NOR Gate (IC 7402)

## Theory

NOR gate: Y = (A+B)' = A'·B'  [De Morgan's Theorem]

NOR is the dual universal gate. Tying inputs gives NOT, chaining two NOR gives OR, three NOR gives AND.

## IC 7402 — Quad 2-Input NOR — Full Pin Diagram

IMPORTANT: IC 7402 pin order is REVERSED compared to IC 7400! Output pin comes FIRST per gate.

| IC 7402 — 7402 |  |  |
| --- | --- | --- |
| Pin 1 — Y1 (Gate 1 OUTPUT) | │ | Pin 14 — VCC (+5V) |
| Pin 2 — A1 (Gate 1 Input A) | │ | Pin 13 — Y4 (Gate 4 OUTPUT) |
| Pin 3 — B1 (Gate 1 Input B) | │ | Pin 12 — A4 (Gate 4 Input A) |
| Pin 4 — Y2 (Gate 2 OUTPUT) | │ | Pin 11 — B4 (Gate 4 Input B) |
| Pin 5 — A2 (Gate 2 Input A) | │ | Pin 10 — Y3 (Gate 3 OUTPUT) |
| Pin 6 — B2 (Gate 2 Input B) | │ | Pin 9 — A3 (Gate 3 Input A) |
| Pin 7 — GND (0V) | │ | Pin 8 — B3 (Gate 3 Input B) |
| Quad 2-input NOR |  |  |

WARNING: IC 7402 Gate 1 -> Output=Pin1, InputA=Pin2, InputB=Pin3  (Output FIRST, then Inputs)

## Circuit 1B-1: NOT Gate from NOR (IC 7402)

Formula: Y = A'   |   NOR gates needed: 1   |   IC 7402 needed: 1

Tie both inputs of NOR gate together:

Y = NOR(A,A) = (A+A)' = A'    [since A+A = A]

## Circuit 1B-2: OR Gate from NOR (IC 7402)

Formula: Y = A+B   |   NOR gates needed: 2   |   IC 7402 needed: 1

Step 1: W = NOR(A,B) = (A+B)'

Step 2: Y = NOR(W,W) = (W+W)' = W' = ((A+B)')' = A+B

## Circuit 1B-3: AND Gate from NOR (IC 7402)

Formula: Y = A·B   |   NOR gates needed: 3   |   IC 7402 needed: 1

A·B = (A''·B'') = (A'+B')'  [De Morgan's Theorem]

Step 1: W1 = NOR(A,A) = A'   [Gate 1 as NOT]

Step 2: W2 = NOR(B,B) = B'   [Gate 2 as NOT]

Step 3: Y  = NOR(W1,W2) = (A'+B')' = A·B   [Gate 3]

## Circuit 1B-4: XOR Gate from NOR (IC 7402)

Formula: Y = A xor B   |   NOR gates needed: 5   |   IC 7402 needed: 2

XOR from NOR — 5-gate implementation:

Step 1

N1=(A+B)'

Step 2

N2=(A+N1)'

Step 3

N3=(B+N1)'

Step 4

N4=(N2+N3)'

এখানে N4 = XNOR

Step 5

N4 কে invert করো

XOR=(N4+N4)'

N1 = NOR(A,B)

N2 = NOR(A,N1)

N3 = NOR(B,N1)

N4 = NOR(N2,N3)    ← XNOR

XOR = NOR(N4,N4)

XOR = NOR(N4,N4) Y  = NOR(W3, W4) = (A·B'+A'·B)' ... need NOT: Y = NOT(NOR(W3,W4))'

Corrected: Y = NOR(NOR(A',B), NOR(A,B')) using 4 NORs, then NOT = 5th NOR tied to itself

Circuits ss:

File Link:  file

YouTube link:   youtube1		Youtube2  		Youtube3

EXPERIMENT 2: Circuit from Minterms Using K-Map

## Objective

Simplify Boolean functions given as minterm lists using 3-variable and 4-variable K-Maps, then implement with ICs.

## 2.1 Three-Variable K-Map Theory

Variables: A, B, C  |  Total cells: 2^3 = 8  |  Minterm range: m0 to m7

### K-Map Layout (Gray Code columns):

BC

00           01         11          10

┌────┬────┬────┬────┐

A = 0	 │ m0     │ m1     │ m3      │ m2     │

├────┼────┼────┼────┤

A = 1	 │ m4     │ m5     │ m7      │ m6     │

└────┴────┴────┴────┘

### Minterm Table (3-variable):

| Minterm | Binary ABC | Expression | Minterm | Binary ABC | Expression |
| --- | --- | --- | --- | --- | --- |
| m0 | 000 | A'B'C' | m4 | 100 | AB'C' |
| m1 | 001 | A'B'C | m5 | 101 | AB'C |
| m2 | 010 | A'BC' | m6 | 110 | ABC' |
| m3 | 011 | A'BC | m7 | 111 | ABC |

### Group Size Rule:

| Group Size | Literals Eliminated | Remaining Literals |
| --- | --- | --- |
| 1 (no grouping) | 0 | 3 |
| 2 | 1 | 2 |
| 4 | 2 | 1 |
| 8 (all cells) | 3 | 0 (F=1) |

## 2.2 Four-Variable K-Map Theory

Variables: A, B, C, D  |  Total cells: 2^4 = 16  |  Minterms: m0 to m15

### 4-Variable K-Map Layout:

CD

00                     01                 11                   10

┌───────┬───────┬───────┬───────┐

AB=00│  m0            │  m1             │  m3             │  m2            │  A=0, B=0

├───────┼───────┼───────┼───────┤

AB=01│  m4             │  m5            │  m7             │  m6             │  A=0, B=1

├───────┼───────┼───────┼───────┤

AB=11│  m12           │  m13           │  m15          │  m14           │  A=1, B=1

├───────┼───────┼───────┼───────┤

AB=10│  m8             │  m9             │  m11           │  m10          │  A=1, B=0

└───────┴───────┴───────┴───────┘

### Complete 4-Variable Minterm Reference:

| m# | ABCD | m# | ABCD | m# | ABCD | m# | ABCD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m0 | 0000 | m4 | 0100 | m8 | 1000 | m12 | 1100 |
| m1 | 0001 | m5 | 0101 | m9 | 1001 | m13 | 1101 |
| m2 | 0010 | m6 | 0110 | m10 | 1010 | m14 | 1110 |
| m3 | 0011 | m7 | 0111 | m11 | 1011 | m15 | 1111 |

### Example: use ChatGPT and happy ending

# EXPERIMENT 3: Parity Generator and Checker Circuit

## Theory

Parity Bit: Extra bit added to data so total count of 1s satisfies a rule.

Even Parity: Total 1s (data + parity) = Even number

Odd Parity:  Total 1s (data + parity) = Odd number

XOR property: Result=1 if ODD number of inputs are 1. Perfect for parity!

## IC 7486 — Quad 2-Input XOR — Full Pin Diagram

| IC 7486 — 7486 |  |  |
| --- | --- | --- |
| Pin 1 — A1 (Gate 1 Input A) | │ | Pin 14 — VCC (+5V) |  |
| Pin 2 — B1 (Gate 1 Input B) | │ | Pin 13 — A4 (Gate 4 Input A) |  |
| Pin 3 — Y1 (Gate 1 Output) | │ | Pin 12 — B4 (Gate 4 Input B) |  |
| Pin 4 — A2 (Gate 2 Input A) | │ | Pin 11 — Y4 (Gate 4 Output) |  |
| Pin 5 — B2 (Gate 2 Input B) | │ | Pin 10 — A3 (Gate 3 Input A) |  |
| Pin 6 — Y2 (Gate 2 Output) | │ | Pin 9 — B3 (Gate 3 Input B) |  |
| Pin 7 — GND (0V) | │ | Pin 8 — Y3 (Gate 3 Output) |  |
| Quad 2-input XOR |  |  |

## 3-bit Even Parity Generator

Inputs: A, B, C  |  Output: P = A xor B xor C

Gate 1: W = A xor B  (XOR first two bits)

Gate 2: P = W xor C  (XOR with third bit)

### Truth Table:

| A | B | C | 1s Count | W=A xor B | P=W xor C | Transmitted ABCP |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0(even) | 0 | 0 | 0000 |
| 0 | 0 | 1 | 1(odd) | 0 | 1 | 0011 |
| 0 | 1 | 0 | 1(odd) | 1 | 1 | 0101 |
| 0 | 1 | 1 | 2(even) | 1 | 0 | 0110 |
| 1 | 0 | 0 | 1(odd) | 1 | 1 | 1001 |
| 1 | 0 | 1 | 2(even) | 1 | 0 | 1010 |
| 1 | 1 | 0 | 2(even) | 0 | 0 | 1100 |
| 1 | 1 | 1 | 3(odd) | 0 | 1 | 1111 |

## 3-bit Even Parity Checker

Boolean Equation

Error output: E=A⊕B⊕C⊕P

Interpretation

- E = 0 → No Error

- E = 1 → Error Detected

## Truth Table

| A | B | C | P | E |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 |

Odd Parity Generator and Checker Circuit

Podd​=(A⊕B⊕C)′

Circuit

- 2টি XOR Gate

- 1টি NOT Gate

Odd Parity Checker

Received bits: A, B, C এবং parity bit P

Checker-এর output E:

E=A⊕B⊕C⊕P

Interpretation (Odd Parity)

| E | Meaning |
| --- | --- |
| 1 | No Error |  |
| 0 | Error Detected |

কারণ Odd parity-তে সঠিক data এ মোট 1-এর সংখ্যা odd হবে, তাই XOR-এর ফল 1 হবে।

File Link: file

EXPERIMENT 4: BCD to Excess-3 Conversion Circuit

## Objective

Convert 4-bit BCD input (A,B,C,D) to Excess-3 output (W,X,Y,Z) using K-Map and standard ICs.

## Truth Table — BCD to Excess-3(IC: OR, AND, XOR, NOT)

| Dec | A | B | C | D | W | X | Y | Z | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0+3=3 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1+3=4 |
| 2 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 2+3=5 |
| 3 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 3+3=6 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 4+3=7 |
| 5 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 5+3=8 |
| 6 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 6+3=9 |
| 7 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 7+3=10 |
| 8 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 8+3=11 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 9+3=12 |
| 10-15 | X | X | X | X | X | X | X | X | Invalid BCD |

File Link: file

YouTube link:   youtube

YouTube link:                              	File Link:

# EXPERIMENT 5A: Full Adder Implementation

## Objective

Design and implement a Full Adder that computes A + B + Cin producing Sum (S) and Carry-out (Cout).

## Truth Table

| A | B | Cin | Sum S | Carry Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Boolean Expressions

🔹 Sum:

S=A⊕B⊕Cin

🔹 Carry-out:

Cout=AB+ACin+BCin

Alternative form:

Cout=AB+Cin(A⊕B)

YouTube link:    youtube

File Link:   file

# EXPERIMENT 5B: Full Subtractor Implementation

## Objective

Design and implement a Full Subtractor computing A - B - Bin, producing Difference (D) and Borrow-out (Bout).

## Truth Table

| A | B | Bin | Difference D | Borrow Bout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

YouTube link: youtube

File Link:	file